Dr. Mahalingam College of Engineering and Technology, Pollachi.

(An Autonomous Institution affiliated to Anna University)

M.E DEGREE EXAMINATION, MARCH – JUNE 2015

Second Semester – Structural Engineering

140ST0201 Finite Element Techniques - Question Paper Code : 01223-E

KEY to Question paper.

PART- A

- Examples for plane stress problem: Thin plate subjected to in-plane loads

Examples for plane stress problem: A dam. (A prismatic long body subjected to body restrained longitudinal movements at the ends subjected constant load along its length).

- The principle of minimum of total potential energy states that, of all possible kinematically admissible displacement fields, the actual solution to the problem is one which has minimum total potential energy.

- In finite element method, Shape functions are, expressions in-terms of coordinates of a point within an elements, used to form an interpolation function to find the value of field variable or geometric variable with the help of nodal values of variables(Field or .geometric).

- AEL1-1-11u1u2=F1F2 Where E, A, L are Young’s modulus of the material, cross sectional area and length of bar element respectively.

u1 and u2 are axial nodal displacements at node 1 and node 2 respectively.

F1 and F2 are axial force applied at node 1 and node 2 respectively.

- In super parametric elements, number of nodes used to define geometry will be more than the number of nodes used for defining field variables (displacement, temperature etc). Whereas in subparametric elements, number of nodes used to defined geometry will be less than that of number of nodes used for defining field variables.

- In case of curved lines or curved elements, shape functions are, first, expressed in terms of natural coordates (ξ, η) [parent element] and interpolation functions are formed for finding corresponding point mapped element (global coordinates). Similary, shape functions used to form an interpolation function for defining field variable (Displacement, temperature etc.). If shape functions used in defining geometry and field variables, the it is called isoparametric element. Subsequently, using these functions, method of arriving element properties such as stiffness matrix, load vector etc is called Isoparametric formulation.

- A) Methods to solve governing differential equation

Weighted Residual methods

Collocation method

Subdomain method

Galerkin method

B) Methods based on Principle of total minimum potential energy

Ritz Method

Rayleigh-Ritz method

- Displacement/Stiffness Method

Equilibrium/Force Method

- Sway of tall buildings

Galloping of bridges

(Vibration of cantilever beams, spring-mass system – any two of student choice)

- Jacobi method, Hessenberg reduction, QR method, Divide and conquer, (Any two)

PART- B

QUESTION 11

Let consider po/m uniformly distributed load is acting over entire span.

E is the Young’s modulus and I is moment of inertia of the beam of length L.

The following function for deflection is satisfying boundary conditions.

(In Question, Particular trial function is not mentironed. Hence Polynomial function also can be considered if that satisfies boundary condition).

=5poL4384EI

PART- C

QUESTION 12(a)

QUESTION 12(b)

Given structure is deficient structure. If students followed the steps marks may be awarded suitably

QUESTION 13(a)

8 noded isoparametric solid element (trilinear element)

The definition of ξ, η and ζ can be now be made more precise:

ξ goes from −1 on  face 1 4 8 5 to +1 on face 2 3 7 6

η goes from −1 on  face 1 2 6 5 to +1 on face 3 4 8 7

ζ goes from −1 on face  1 2 3 4 to +1 on face 5 6 7 8

The center of a face is the intersection of the two medians.

Thus coordinate of nodes are:

QUESTION 13(b)

QUESTION 14(a)

Triangular Plate Bending Element

QUESTION 14(b)

STIFFNESS MATRIX OF THE SHELL ELEMENT