Dr. Mahalingam College of Engineering & Technology

Pollachi- 642003

ASSIGNMENT– 2 – Questions

Class & Branch 	: II B.E. -  CIVIL	Max. Marks	: 100

Sub Name	: APPLIED HYDRAULIC ENGINEERING	Time 	: 3 hrs

Semester	: IV	Assigned Date	: 23-MAR-2009

Due Date 	: 23-MAR-2009

Venue	: B308

PART – A (11 x 2 = 22 Marks)

Answer all questions :

No hints.

PART – B ( 6 x 13 = 78 Marks)

No hints for 37 marks.

Hints for remaining 41 marks.

Determine the normal depth in a trapezoidal channel with bottom width ‘B’ m and side slope ‘H1’ horizontal : ‘V1’ vertical when it carries ‘Q’ m3/s of water discharge at a bed slope of 1 in ‘X’. Assume Manning’s roughness coefficient as ‘n’. 	(8)

The specific energy for a ‘B’ m wide rectangular channel is ‘E’. If the rate of flow of water through the channel is ‘Q’ m3/s. Determine the alternate depths of flow.			(7)

Determine approximate water surface slope (m per km) at a point in a rectangular channel in which the discharge is ‘Q’ m3/s, the bed width is ‘B’ m, the depth of flow is ‘y’ m and the bed slope is 1 in ‘X’. Assume mannings roughness coefficient as ‘n’ and take energy coefficient alpha equal to unity. 	(9)

Water flows at a steady and uniform depth of ‘y‘ m in an open channel of rectangular cross - section having base width equal to ‘B’ m and laid at a slope of 1 in ‘X’. It is desired to obtain critical flow in the channel by providing a hump in the bed. Calculate hump height and sketch the flow profile. Consider the value of Mannings rugosity (roughness) coefficient n for the channel surface. 	(13)

A trapezoidal channel with side slope 1:2 and bed slope 1:750 has a ‘B’m wide bed composed of sand (n1) and sides concrete (n2). Estimate the discharge when the depth of flow is ‘y’m 	(4)