Dr. Mahalingam College of Engineering & Technology

Pollachi- 642003

CONTINUOUS ASSESSMENT TEST – II

Questions and Answers

Class & Branch 	: II B.E. -  CIVIL	Max. Marks	: 50

Sub Name	: APPLIED HYDRAULIC ENGINEERING	Time 	: 1½ hrs

Semester	: IV	Test date : 31-MAR-2009

Answers released date : 03-APR-2009

PART – A (10 x 2 = 20 Marks)

Answer all questions :

- List the factors affecting mannings coefficient ‘n’?

Some important factors affecting manning’s coefficient are: 
(a) surface roughness, 
(b) vegetation, 
(c) cross-section irregularity and 
(d) irregularity alignment of channel.

- Define conveyance of the channel?

Conveyance is expressed as the discharge capacity of the channel per unit longitudinal slope. K=QSo

- Explain the first and second hydraulic exponents.

First hydraulic exponent: When expressing relationship between section factor Z and flow depth as

Z2=A3T=C1yM, the exponent M is called first hydraulic exponent.

Second hydraulic exponent: Relating conveyance factor K and flow depth, it can be approximately written as K2=C2yN. In this expression ‘N’ is called the second hydraulic coefficient.

- List the condition for hydraulically efficient trapezoidal channel.

With the slope, roughness coefficient and area of flow fixed, a minimum perimeter section will represent the hydraulically-efficient section as it conveys the maximum discharge.

Condition for Hydraulically efficient section: When area of section A and side slope m are fixed, the sum of side slope lengths of the cross-section should be equal to top width of the flow. Thus the proportions of a hydraulically efficient trapezoidal channel section will be such that a semi-circle can be inscribed in it.

Condition for Hydraulically most efficient section: Along with the above condition, m should be equal to 1/√3 ie. Side slope at 60° with the horizontal. Thus the hydraulically most efficient trapezoidal section becomes one-half of a regular hexagon.

- What is the critical slope in the case of uniform flow?

The slope of a channel which carries a given discharge as a uniform flow at the critical depth is called the critical slope

- What are the assumptions involved in the analysis of gradually varied flow

- Write the dynamic equation of GVF which involves geometry properties and discharge.

dydx=S0−Sf1−Q2TgA3

- Explain back water curves?

### Obstructions to flow, such as weirs, Dams, structures and natural features, such as bends increase the depth of flow and makes water surface not parallel to bed slope. The longitudinal profile of these kind of water surfaces (gradually varied flow) is called as back water curve.

- What are the possible profiles in a steep sloped open channel flow?

S1, S2 and S3 profiles.

All these profiles occur in the steep sloped channel.

S1 profile - The  S1  profile is produced when the flow from a steep channel is obstructed by a weir or dam.

S2 profile - Profiles of the S2 type occur at a break of grade from mild slopes to steep slope.Generally  S2  profiles are of short length.

S3 profile - Flow from a sluice gate with a steep slope on its downstream is of  the S3 type. The S3 curve also results when a flow exists from a steeper slope to a less steep slope.

- What type of profile exists when a channel has break in grade from steep channel to a mild channel? Sketch the water profile

The  S1  profile is produced when channel has break in grade from steep channel to a mild channel.

PART – B ( 3 x 10 = 30 Marks)

Answer any three of the following

- (a) Derive the Chezy’s formula from basic principle for uniform flow through open channel and compare it with manning’s formula. 	(5)

By definition there is no acceleration in uniform flow. By applying the momentum equation to a control volume encompassing sections 1 and 2, distance L apart, as shown in Fig. 3.1,

P1+Wsinθ−Ff−P2=M2−M1. 	Eq(13.1)

where P1 and P2    are the pressure forces and  M1   and M2 are the momentum fluxes at section 1 and 2 respectively.   W  = weight of fluid in the control volume and      Ff = shear force at the boundary.

Since the flow is uniform, P1=P2 and M1=M2

Also, W=γAL and Ff=τ0PL

where   τ0  =  average shear stress on the wetted perimeter of length   P   and      γ  = unit weight of water. Replacing  sinθ  by   S0(= bottom slope), Eq. (13.1) can be written as γALS0=τ0PL.

Or τ0=γAPS0=γRS0	Eq(13.2)

where  R=A/P  is defined as the hydraulic radius.

R is a length parameter accounting for the shape of the channel. It plays a very important role in developing flow equations which are common to all shapes of channels.

Expressing the average shear stress  τ0 as τ0=kρV2.

where k is a coefficient which depends on the nature of the surface and flow parameters, Eq. (13.2) is written as

kρV2=γRS0 leading to V=CRS0. 	Eq(13.3)

where   C=γρ1kis a coefficient which depends on the nature of the surface and the flow. Equation (13.3) is known as the Chezy formula

11 (b) A concrete lined trapezoidal channel (n = 0.015) is 8 m wide and has a side slope of 2 horizontal : 1 vertical. The longitudinal; slope is 0.006. Estimate the normal depth in this channel for a discharge of 40 m3/s	(5)

B=8m , Q = 40 m3/s, m=2

So = 0.006 and n=0.015.

Yo = ?

According manning’s formula

Q=1nA.R23So

Where Q is the discharge, A is the area of flow, R=A/P, P is the wetted perimeter. So, it can also be written as Q=1nA53P23So

A= (8+2yo)yo and P=(8+222+1  yo)=(8+4.4721 y0)

Substituting value of A, P, Q, n and So.

40=10.015(8+2yo)53yo53(8+4.4721yo)230.006

(8+2yo)53yo53(8+4.4721yo)23=7.7460

Solving the above equation, normal depth 0.9403 m

- A standard lined trapezoidal section to carry a discharge of 100 m3/s at a slope of 1:1000. The side slopes are to be 1.5 H : 1 vertical and the manning’s ‘n’ can be taken as 0.015. What bottom width is needed to have a full supply depth of 2.00m?	(10)

y0=2.00m,

Q = 100 m3/s,

m=1.5, So = 0.001

and n=0.015.

B = ?

m = cotθ =1/tanθ

θ = arctan(1/m) = 0.5880 Rad

ε = m+θ = 1.5+0.5880 = 2.0880

A = B×y0+2×π×y022πθ+2×y0y0tanθ=B×y0+y02θ+2×y0y0tanθ2

=B+y0θ+y0tanθy0=B+y0(θ+m)y0

Hence A = (B+2.0880 * 2)*2 = 2B+8.352

P = B+2y0θ+2×y0tanθ=B+2y01tanθ+θ=B+2y0m+θ

P=B+2*2*2.0880=B+8.352

From manning’s formula, We know that,

A53P23=nQSo

(2B+8.352)53(B+8.352)23=0.015∗1000.001

(2B+8.352)53(B+8.352)23=47.4342

Solving this, we get,

B= 13.0864m

- The compound section shown in fig. 1. Assume n=0.02 and So = 0.0009 for all parts of the perimeter. Find the discharge a) using Posey’s method, b) Zero shear method c) Total section method.	(10)

1

3

2

10.0 m

10.0 m

5.0 m

3.0 m

1.5 m

1.5 : 1

1.5 : 1

1 : 1

1 : 1

In total section method, the discharge is calculated by considering the whole section as one unit.

In the zero-shear method, actual area and actual perimeter of the each portion of the channel to compute the partial discharge. Then discharge is calculated as sum of all the partial discharges.

In the case of Posey’s method, imaginary shear interfaces are accounted in calculating perimeter for deep portion only.

Area A1 = A3 = 1.5 *[10 + (10+1.5*1.5)]/2=16.6875 m2

Actual perimeter P1 = P3 = 10+sqrt(1.5^2+1)*1.5=12.7042 m

Area A1 = 3*(5 + 3) + 11*1.5=40.5 m2

Actual perimeter P2 = 5+2*sqrt(2)*3=13.4853 m

Perimeter with imaginary shear interface = 13.4853+3=16.4853 m

| section | Area | Perimeter | Perimeter for Posey’s method |
| --- | --- | --- | --- |
| 1 | 16.6875 m2 | 12.7042 m | 12.7042 m |
| 2 | 40.5 m2 | 13.4853 m | 16.4853 m |
| 3 | 16.6875 m2 | 12.7042 m | 12.7042 m |

Total area = 2*16.6875+40.5 = 73.8750 m2

Total perimeter = 2*12.7042+13.4853  = 38.8937m

Discharge by Total section method

Q = 1/0.015*(73.875^(5/3)/38.8937^(2/3))*sqrt(0.0009)

Qw=226.6068 m3/s

Discharge by zero shear method

Q1 =Q3 = 1/0.015*(16.6875^(5/3)/12.7042^(2/3))*sqrt(0.0009)

Q1=40.0298 m3/s

Q2 = 1/0.015*(40.5^(5/3)/13.4853^(2/3))*sqrt(0.0009)

Q1=168.6092 m3/s

Total Q = Q1 + Q2 + Q3 = 2*40.0298 + 168.6092 = 248.6688 m3/s

Discharge by Posey’s method

Q1 =Q3 = 1/0.015*(16.6875^(5/3)/12.7042^(2/3))*sqrt(0.0009)

Q1=40.0298 m3/s

Q2 = 1/0.015*(40.5^(5/3)/ 16.4853^(2/3))*sqrt(0.0009)

Q1=147.4768 m3/s

Total Q = Q1 + Q2 + Q3 = 2*40.0298 +147.4768= 227.5364 m3/s

- How surface profiles of Gradually Varied Flow are classified and explain them with sketches. 	(10)

Classification of GVF water surface profiles.

(a) Type M Profiles

The most common of all GVF profiles  M1   type, which is a  subcritical-flow condition. Obstructions to flow, such as weirs, Dams, structures and natural features, such as bends, produce  M1   backwater curves.

The  M2   profiles occur at a sudden drop in bed of the channel, at constriction type of transitions and at the canal outlet into pools

The  M3 type of profile occurs where a supercritical stream enters a mild- slop channel. The flow leading from a Spillway or sluice gate to a mild slope forms a typical example

(b) Type  S  Profiles

The S1 profile is produced when the flow from a steep channel is terminated by a deep pool created by an obstruction, such as a weir or dam. At the beginning of the curve, the flow changes from the normal depth (supercritical flow) to subcritical flow through a hydraulic jump.

Profiles of the S2 type occur at the entrance region of a steep channel leading from a reservoir and at a break of grade from mild slopes to steep slope. Generally  S2  profiles are of short length.

Flow profile from a sluice gate with a steep slope on its downstream is of  the  S3   type. The  S3   curve also results when a flow exists from a steeper slope to a less steep slope

(c) Type  C  Profiles

C1 and C3 profiles are very rare and are highly unstable.

(d) Type H Profiles

A horizontal channel can be considered as the lower limit reached by a mild slope as its bed slope becomes flatter. It is obvious that there is no region 1 for a horizontal channel as yo  ∞. The  H2  and  H3  profiles are similar to  M2   and   M2 profiles respectively.

(e) Type A Profiles

Adverse slopes are rather rare and  A2  and  A3 curves are  similar to  H2    and  H3 curves respectively. These  profiles are of very short length.

- A rectangular channel has three reaches A, B, C connected in series in the same order with the following geometric properties:

| Reach | Bed Width B | Bed slope So | n |
| --- | --- | --- | --- |
| A | 4 | 0.001 | 0.015 |
| B | 4 | 0.009 | 0.012 |
| C | 4 | 0.006 | 0.015 |

For a discharge of 23 m3/s through this channel, sketch the resulting water-surface profiles. The length of the reaches can be assumed to be sufficiently long for the GVF profiles to develop fully.	(10)

Critical depth yc = (Q^2/(g*B^2))^(1/3) = (23^2/ (9.81*4^2))^(1/3) = 1.4993 m

This yc is applicable for all reaches. But yo is different for each reach.

For uniform flow, A^(5/3)/P^(2/3)=Q n /Sqrt(so)

This reduces for rectangular channel to

(By)^5/3/(B+2*y)^(2/3)=Q n / sqrt(so),

In this problem,

10.0794y^(5/3)/(4+2y)^(2/3)=23*n/sqrt(so)

For reach A

10.0794y0^(5/3)/(4+2y0)^(2/3)=23*0.015/sqrt(0.001)

Solving we get y0A =  2.5327 m

For reach B

10.0794y0^(5/3)/(4+2y0)^(2/3)=23*0.012/sqrt(0.009)

Solving we get y0B =  0.9673 m

For reach C

10.0794y0^(5/3)/(4+2y0)^(2/3)=23*0.015/sqrt(0.006)

Solving we get y0B = 1.3037 m

NDL

CDL

2.5327 m

0.9673m

1.3037 m

1.4993 m