Dr. Mahalingam College of Engineering & Technology

Pollachi- 642003

CONTINUOUS ASSESSMENT TEST – II

Class & Branch 	: II B.E. -  CIVIL	Max. Marks	: 50

Sub. Code & Name	: Mechanics of Fluids	Time 	: 1½ hrs

Semester	: III	Date	:    -OCT-2009

PART – A (10 x 2 = 20 Marks)

Answer all questions :

- What is the Eulerian method of describing the fluid flow?

- Explain the terms a). path line, b). streak line, c). stream line and d). stream tube.

- Name the types of fluid flow.

- Distinguish between steady flow and uniform flow.

- Find the velocity potential function Φ= x2-y2, find the velocity components at the point (4,5).

- Define the equation of continuity. Write the continuity equation for a 3-D flow.

- Define the convective and local accelerations.

- Name the forces present in a fluid flow? What are forces are considered for the Euler’s equation of motion?

- What is the difference between pitot tube and pitot-static tube?

- Explain the working principle of current meter.

PART – B ( 3 x 10 = 30 Marks)

Answer any three of the following

- Define the stream function and the potential function in two-dimensional ideal fluid flow and show that stream lines and equipotential lines always intersect at right angles.

- Derive the Euler’s equation of motion of a fluid. Also State the assumptions and derive Bernoulli’s Equation from it.

(P.T.O)

- A pipe line carrying oil of specific gravity 0.87, changes in diameter from 200mm diameter at a position A to 500mm diameter at a position B which is 4m at a higher level than A. If the pressure at A and B are 10N/cm2 and 6 N/cm2 respectively and the discharge is 200 litres/sec, determine the loss of head and direction of flow.

- Explain the principle of venturi meter with a neat sketch? Derive an expression for the discharge through venturi meter.

- A 20cm (Inlet diameter) x 10cm (throat diameter) venturimeter is inserted in a vertical pipe carrying oil of sp. gr. 0.8. The flow of oil is in upward direction. The difference of levels between the throat and inlet section is 50cm. The oil-mercury differential manometer gives a reading of 300mm of mercury. Find the discharge of oil through the pipe. Take Cd = 0.95.