Dr. Mahalingam College of Engineering & Technology

Pollachi- 642003

CONTINUOUS ASSESSMENT TEST – III

Class & Branch 	: III B.E. - CIVIL	Max. Marks	: 50

Sub. Code & Name	: STRENGTH OF MATERIALS	Time 	: 1½ hrs

Semester	: V	Date	: 26-OCT-2009

PART – A (10 x 2 = 20 Marks)

Answer all questions:

- What is slenderness ratio or buckling factor?

- What is the difference between column and strut?

- What are the factors affect the column strength?

- What are the assumptions followed in the Euler’s formula?

- Give the ratios of equivalent length and actual length of columns with various end conditions.

- Define a thick cylinder?

- What do you mean by thick compound cylinders?

- What are the principal stresses?

- List out different theories of failure.

- Define volumetric strain

PART – B ( 3 x 10 = 30 Marks)

Answer any three of the following

- Using Euler's theory, derive the buckling load for the columns with one end Fixed and other end hinged.

- A column with one end hinged and the other end fixed has a length of 
5 m and a hollow circular cross section of outer diameter 100 mm and wall thickness 10 mm. If rankine constant = 1/1600, E=1.2x105 N/mm2 and crushing strength = 550N/mm2, find the safe load that the column may carry with a factor of safety of 2.5 according to Euler theory and Rankine-Gordon theory. If the column is hinged on both ends, find the safe load according to the two theories.

- A cylinder of outer diameter 300 mm and inner diameter 250 mm shrunk over another cylinder of outer diameter slightly more than 250 mm and inner diameter 200 mm to form a compound cylinder. The shrink fit pressure is 10 N/mm2. If an internal pressure of 50 N/mm2 is applied to the compound cylinder, find the final stresses across the thickness. Draw sketches showing their variations.

- Find the thickness of metal necessary for a cylindrical shell of internal diameter 150 mm to withstand an internal pressure of 50 N/mm2. The maximum hoop stress in the section is not to exceed 150 N/mm2.

- Determine the principal stresses and principal directions for the following 3D- stress field: 954583437