Dr.Mahalingam College of Engineering & Technology

Pollachi  - 642 003

# ASSIGNMENT - III

## Class & Branch:  III B.E - CIVIL	Max Marks: 100

Sub.Code & Name: STRUCTURAL ANALYSIS -I	Time:

Semester: V	Due Date : 05.10.2010 @ 1:00PM

Part-A (20´2=40 Marks)

Answer all questions:

- Define sway and mention any 4 reasons due to which sway may occur in portal frames.

- Who introduced slope-deflection method of analysis and when?

- What are the assumptions made in slope-deflection method?

- State the limitations of slope deflection method.

- Why is slope-deflection method called a ‘displacement method’?

- Write down the general slope-deflection equations and explain each term.

- What is the basis on which the sway equation is formed for a structure?

- What is rigid joint in the portal frames?

- Write the fixed end moments for a beam carrying a central clockwise moment M.

- Write the fixed end moments for a beam of span of  l subjected to a concentrated load at 0.25 l from left end.

- Define: Moment distribution method.( Hardy Cross method).

- What is the difference between absolute and relative stiffness?

- Define stiffness factor and distribution factor

- Define: Carry over moment and Carry over factor

- What is the sum of distribution factors at a joint?

- Three members meeting at a rigid joint carry moments 5 kNm(clockwise), 10kNm (clockwise) and 9kNm (counter-clockwise) . What moment should be applied at that joint for achieving the equilibrium? What is the name of this activity in the moment distribution method?

- Distribute a moment of 6kNm at a joint among three members whose relative stiffnesses are 1/6, 1/3 and 1/2.

- Sketch the equilent for unsymmetric udl, acting only half of the span of symmetrical 2 span continuous beam, interms of symmetric and anti-symmetric loads.

- What are symmetric and antisymmetric quantities in structural behaviour?

- What is reduced structure with respect to symmetric structure but unsymmetrical loading?

### Part-B (4*15=60 Marks)

Answer all Four

3m

A

B

3m

C

D

10 kN/m

Fig. 1

60 kN

2m

4m

( 2Io )

( Io )

(Io)

Analyse the frame shown in fig 1. by the slope deflection method and sketch the bending moment diagram.

(15)

A

B

3m

C

D

Fig. 2

42 kN

2m

4m

(Io )

( Io )

(2Io)

6m

6 kN/m

5 kN/m

Analyse the portal frame shown in fig. 2 using slope deflection method and sketch the bending moment and shear force diagram.	(15)

3m

A

B

2m

C

D

40kN/m

4m

(3I)

(2I)

(2I)

80 kN

2m

Fig. 3

Analyse the structure loaded as in fig. 3. by the moment distribution method and sketch the bending moment and shear force diagram..	(15)

Fig. 4

A

B

C

D

50 kN/m

(2I)

( I )

3m

3m

4m

(I)

Analyse the portal frame loaded as shown in fig. 4 by the moment distribution method and sketch the BMD and SFD.	(15)

----------  End  -----------