Dr.Mahalingam College of Engineering & Technology

Pollachi  - 642 003

# CONTINUOUS ASSESSMENT TEST I

## Class & Branch:  III B.E - CIVIL	Max Marks: 50

Sub.Code & Name: STRUCTURAL ANALYSIS - II	Time:  9:30 to 11:00AM

Semester: VI	Date : 21.01.2011 (FN)

Part-A (10´2=20 Marks)

Answer all questions:

- Define flexibility coefficient

- Write down the flexibility matrix for a bar element and a beam element.

- Define local and global coordinates

- Explain transformation matrix.

- Write the relationship between the elemental displacements with system displacements?

- What is the formula for the degree of static indeterminacy in case of 
1). Truss 2). Frames?

- What is the degree of static indeterminacies (Total, internal and External indeterminacies) of rigid jointed frame shown in fig. 2.

- What is a primary structure?

- Define stiffness coefficient

- Write down the stiffness matrix for the beam element.

### Part-B (3*10=30 Marks)

Answer any Three

- Explain the steps involved in of flexibility matrix method and stiffness matrix method side by side in the form of tabular column.	(10)

- Generate flexibility matrix [a] of the structure with two coordinates indicated in the Fig. 1.	(10)

L

C

E

1

0.7L

EI=16 for all members

D

B

0.8L

2

Fig. 1

(P.T.O)

A

Fig. 2

4 m

100kN

6m

A

D

C

B

2m

50 kN

Analysis the portal frame shown in Fig. 2 using  the flexibility matrix method if IAB=2Io; IBC = Io ; ICD=2Io.	(10)

- Using the matrix flexibility method, find the member forces in the pin-jointed truss shown in Fig. 3. Cross sectional areas of members are : 1000mm2 and E=200kN/mm2.	(10)

Fig. 3

3 m

A

B

C

E

D

100 kN

4m

4m

--------------------------------- End ---------------------------------