turDr. Mahalingam College of Engineering & Technology

Pollachi- 642003

ASSIGNMENT– I

Class & Branch 	: IV B.E. -  CIVIL

Sub. Code & Name	: STRUCTURAL ANALYSIS-2	ASSIGNED DATE :19-JUL-2010

Semester	: VII	Due Date	: on or before 26-JUL-2010 9:15AM

PART – A

Answer all questions : (2 Marks Each)

- Write down the compatibility equation used in flexibility matrix method

- Define flexibility coefficient

- Write generalized forces-generalized displacements relationship for a simple structure.

- Write down the flexibility matrix for 2D beam element

- What is a transformation matrix?

- Define local and global coordinates

- Find the degree of static indeterminacy of rigid jointed frame shown in fig.

- What is a primary structure?

- Define stiffness coefficient?

- What is the formula for the degree of static indeterminacy in case of 1). Truss 2). Frames.

PART – B

Answer ALL (10 marks Each)

X = last 2 digit of your roll No.

- Explain the steps involved in of flexibility matrix method and stiffness matrix method side by side.	(10)

2+(X/30)m

2+(X/30) m

3+(x/30)m

Fig. 2

80KN

Analysis the rigid jointed portal frame shown in fig. 2 by 1). the flexibility matrix method and 2). the stiffness matrix method.	(15)

Using the matrix flexibility method, find the member forces in the pin-jointed truss shown in fig. 2. Cross sectional areas of members are : AE and FD = 250mm2; EF, BF and EC = 1875mm2; AB, BC, CD, EB and FC = 1250mm2 and E=200kN/mm2.	(10)

E

F

Fig. 2

3+(x/40)m

A

B

C

D

X kN

2+(X+40)m

2+(X/40) m

2+ (X/40)m

1+(X/30)m

1+(X/30)m

4+(X/30)M

2I

I

I

A

B

C

D

Draw bending moment and shear force diagram for the frame shown in fig. 4. Relative I values are indicated along the members. E=200x106 kN/m2 and I = 300x10-6 m4. Solve separately by 1). matrix flexibility method and 2). stiffness matrix method. 	(15)

X kN

Fig. 3

--------------------------------- End ---------------------------------