Motivated Study - 2

Assigned Date: 02/11/2009 	Due Date: on or before 07/10/2009  9:00AM

Class & Branch 	: III year B.E. -  CIVIL

Sub. Code & Name	: STRENGTH OF MATERIALS

Semester	: V

TOTAL 10 Marks

Follow the procedure to compute the given data:

      - Call your last digit of your roll No. (if your roll No. is M07CE68, then take X=68) as X.

      - b1 = (X+100) mm; b2 = (150-X) mm; t1 = (X/10)+10 mm; b2 = 15-(X/10) mm;  h1= 300+ X mm ; Length L1 = 4 + (33-x)/20 m; P1 = 10KN/mm2. P2 = 2KN/mm2.

- A Z section have the following dimension fixed at one end and free at another end subjected to a vertically downward load of P1 at the free end and a horizontal load P2 at the free end the from the left to right. Find the maximum stress and find out Rankine, Tresca, St. Venant, Von-Mises and Beltrami equivalent stresses for checking failure. Draw maximum of these stresses (both the compressive and tensile) along the span.

t1

L1

b1

b2

t2

t111

h1